## ECE4110, Internetwork Programming, QUIZ 2  -  PRACTICE

Spring 2006

Prof. John A. Copeland	Practice for April 11, 2006

Tel.: 404-894-5177

E-Mail: copeland@ece.gatech.edu

RULES.

i	This quiz is not open book.  One original sheet of hand-written notes may be used. Calculators are ok.

ii	Answer all questions and show all work to receive full credit. Use back of sheets only if necessary.

iii	All sub-questions have the same weight.

iv 	Please do not ask the proctors any questions during the exam about exam questions.   Part of the test is understanding the question, as written, without supplemental information.  If you feel additional data is needed to solve the problem, make (and state) an assumption and then work the problem.

v.	This is a time-limited test.  All papers must be turned in 50 minutes after the start.  If you find you are taking more than 10 minutes on a particular problem, move on and come back to that problem after finishing the others.  The Georgia Honor Code applies (see last page).  The actual test will be about 60% as long as this.

Question 1 – TCP with no Congestion.

A 100 Mbps network connection has a 100 ms round trip time (RTT).  A server begins sending maximum-size packets (after the initial SYN-ACK) using Slow Start.  The client ACK's every packet (col. 2) or every-other (col.3), and has a receiver-window which holds 16 maximum-size packets.  Show how many packets are sent in each RTT period below:

| Time Period Start (ms) | No. Segments, when all ACKed | No. Segments, when 1/2 ACKed |
| --- | --- | --- |
| 0 | 1 | 1 |
| 100 |  |  |
| 200 |  |  |
| 300 |  |  |
| 400 |  |  |
| 500 |  |  |
| 600 |  |  |

_____________     How long does it take the server to ramp up to maximum transfer rate (ms).

_____________    If the maximum packet size is 1000 bytes, what is the maximum transfer rate (bytes/second).

How and when is the receiver's maximum segment size (MSS) sent to the sent to the other host?

How?  _____________________________     When _________________________________________

How and when is the receiver's TCP window (WIN) sent to the sent to the other host? ?

How?  _____________________________     When _________________________________________

The last WIN received was 8000, the last ACK number was 12000, and the last TCP segment sent had a SEQ number of 14000.  How many more bytes can be sent?  ___________________

Once those bytes are sent, name three conditions that will allow additional bytes to be sent:

a. __________________________   b.____________________________ c. ____________________________

A TCP host on an Ethernet LAN receives an MSS of 1460 bytes.  It sends IP headers with no options (20 bytes) and TCP headers with 32 bytes (12 bytes for options).  What is the maximum number of data bytes (SMSS) it can send per TCP segment?   _______________________________________________

Question 3 – Bandwidth-Delay Product

a. If the round-trip time (RTT) for a dedicated 16 Mbit/s connection to California is 50 ms, what is the bandwidth-delay product of the link? ________________________(bits).

b. What is the minimum TCP widow size that will allow this channel to be fully utilized?  _______________ (bytes)

c. What will the maximum throughput be if the window is half the value in b. above.       ____________ (bits/s)

d. What will the maximum throughput be if the window is twice the value in b. above.       ____________ (bits/s)

Question 4 – TCP Response to Congestion - Lost and Out-of-Order Packets

Once a TCP segment times out, a variable collision window-size is cwnd is used.  For consistency, we consider cwnd to govern the number of segments sent during the initial Slow Start mode. SMSS (sender maximum segment size) was called segsize in the class slides, MSS in some of the books.  Conditions at the sender may require SMSS to be smaller than the MSS value received for the other TCP host.

What places an upper limit on the maximum number of outstanding (sent, but not ACKed) bytes besides cwnd? _________________________

After a time-out, the value of cwnd drops to _____________ times the maximum segment size (SMSS or segsize).

After a time-out, the value of ssthresh drops to _____________ times the previous value of cwnd.

cwnd then grows in _______________ mode until it reaches ssthresh. which is reduced to  _______ times the value of ____________ before the time-out.

After cwnd reaches ssthresh, cwnd grows linearly by adding SMSS bytes for approximately every __Y__ number of maximum segments sent (ignore the incorrect SMSS/8 term in the slides and book).

What is this linear mode (of cwnd increase) called? __________________________

For the case where the receiver ACKs every packet, Y = _____________________________ (in terms of cwnd, SMSS)

For the case where the receiver ACKs every other packet, Y = _____________________________ (ditto)

What is the maximum value of cwnd? ___________________ (or is it moot, see TCP Illustrated, p. 310, rule 1).

Does cwnd or win (from the receiver) govern the bytes by (sequence number) that can be sent?   ____________________

Question 5 – TCP Fast Retransmission and Fast Recovery

After a TCP sender receives __________ duplicate ACKs it retransmits the first missing segment.

How many additional bytes (at a minimum) can be sent after each additional duplicate ACK is received? ________________

How much does cwnd increase after each additional duplicate ACK is received? ________________

When does Fast Recovery operation start? ________________________________

When FR starts, cwnd is set equal to ___________ x cwnd-before-dup-ACK + _____________  SMSS.

In Fast Recovery,  how many SMSS bytes are added each RTT is all segments are ACKed (do not use SMSS/8 term):

if all segments are ACKed__________________     if every other segment is ACKed?

Question 6– Window Size versus Time

SMSS =1420 bytes, MSS=1420 bytes, RTO = 50 ms, RTT = 30 ms.

What is the Time scale factor, X ? _________________

What is the Bytes scale factor, Y ? _________________

Identify the cwnd growth modes (Slow Start = SS, Collision Avoidance = CA,  Fast Recovery=FR) ?:

Mode A ________________

Mode B ________________

Mode C ________________

Mode D ________________

Mode E ________________

What causes the mode changes ( Start Up = SU, Fast Retransmit = FT, Retransmit Time Out = RTO,  ssthresh reached = SST) ?

Change 0-A ________________

Change A-B ________________

Change B-C ________________

Change C-D ________________

Change D-E ________________

Question 7– Running Average for Calculating the Retransmit Time Out  (use back of page for calculations)

| New Round-Trip Time | A=Average RTT Alpha = 1/8 | Deviation | D = Average Deviation Beta = 1/4 | RTO = A + 4D |
| --- | --- | --- | --- | --- |
| 40 | 30 | 10 | 4 | 26 |
| 50 |  |  |  |  |
| 30 |  |  |  |  |
| 45 |  |  |  |  |

(ignore the 500 ms tick rounding, round results up to 1 ms)

Question 8– Routing,  Distance Vector, RIP

The routing tables for RIP routers A, B, C are shown.

B

A

C

X

| Router A Table |  |  |
| --- | --- | --- |
| Prefix | Distance | Port |
| 128.230. | 2 | X |
| 130.207. | 5 | N |
| 209.196. | 6 | X |
| 24.56. | 9 | X |

| Router B Table |  |  |
| --- | --- | --- |
| Prefix | Distance | Port |
| 128.230. | 2 | X |
| 130.207. | 7 | X |
| 209.196. | 4 | M |
| 24.56. | 9 | X |

| Router C Table |  |  |
| --- | --- | --- |
| Prefix | Distance | Port |
| 128.230. | 2 | X |
| 130.207. | 7 | X |
| 209.196. | 6 | X |
| 24.56. | 7 | P |

Construct the Routing Table for Router X

| Router X Table |  |  |
| --- | --- | --- |
| Prefix | Distance | Port |
| 128.230. |  |  |
| 130.207. |  |  |
| 209.196. |  |  |
| 24.56. |  |  |

Using Split Horizon and Poison Reverse, construct the Updates sent from Router X to A,B,C.

| Update X to A Table |  |  |
| --- | --- | --- |
| Prefix | Distance | Port |
| 128.230. |  |  |
| 130.207. |  |  |
| 209.196. |  |  |
| 24.56. |  |  |

| Update X to B Table |  |  |
| --- | --- | --- |
| Prefix | Distance | Port |
| 128.230. |  |  |
| 130.207. |  |  |
| 209.196. |  |  |
| 24.56. |  |  |

| Update X to C Table |  |  |
| --- | --- | --- |
| Prefix | Distance | Port |
| 128.230. |  |  |
| 130.207. |  |  |
| 209.196. |  |  |
| 24.56. |  |  |

What 2 improvements were added to RIPv1 to make RIP v2

a. ________________________________________________________

b. ________________________________________________________

Question 9– Routing,  Link State, OSPF

Every router (A, B, ..., J) has advertised the costs (delays) to all the other nodes.  For example, node V broadcasts the following route advertisement message: "V, B:29, G:12, U:22, W:26" (In practice, only link states that have changed since the last broadcast are included in the message.)

Based on all the advertisement messages, the network topology and link costs can be mapped.  The letters below represent the nodes (routers) on the network.  The numbers represent costs (delay-times) on the links between them.

These nodes are all routers (no networks) and for simplicity routes have the same cost in both directions.

A-----35-----C-----22-----U-----14-----D

|            |            |             |

45          29            4             5

|            |            |             |

B-----22-----V----56------W-----17-----X ->

|            |            |             |

11           12           16            20

|            |            |             |

F-----13----G-----21-----H-----45------J

A. Using Dijkstra's technique, calculate the cost values of routes from node "A" to the following.

A-[        ], F-[       ], C-[       ], G-[       ], J-[       ]

B.  Cross out (X) the links that are not on the sink tree for node A.

Routers A,B,C,D are connected to Ethernet E.   Show the way this would be diagrammed for an OSPF network diagram?

How are the link-states from a network node advertised? __________________________  What is the costs? __________

Question 9– Routing,  Path Vector, BGP

The Internet is composed of interconnected  ________________ Systems (AS) that are linked by _______BGP routers.

What are the three types of AS, and their defining characteristic?  1. ______________________________________

2. ____________________________________________  3. _____________________________________________

A BGP routing table consists of what two items for every destination ?    1. _________________ 2. _______________

Why is it dangerous to have two border routers which both have the default route set to the other? _____________________________________________________________________________

What information do Autonomous Boundary Routers advertise to their exterior neighbors?

______________________________________________________________________________

To reduce the size of routing tables, network prefixes are _______________ as much as possible.

Question 10 - Protocols for Router Updates

All of these protocols use IP for updates. Indicate multicast IP by "Multi.-" before the transport layer protocols they use.

RIP ______________________  OSPF _________________________  BGP _______________________________

Question 10 - DNS, HTTP, FTP, SMTP, SPAM, SSH questions - see first and last slides of slide set 16 for topics.